# Results and Evaluation

## 1. Experimental Setup (SUMO + NS-3)

This chapter evaluates the proposed Secure Trust-Based V2X Communication with Blockchain Support using SUMO-generated mobility traces converted to NS-2 mobility for ns-3. For each condition, 5 random seeds were used and mean±std values were reported.

Metrics include normalized packet delivery ratio (PDR), average end-to-end delay, throughput, replay/signature drops, ledger trust, blockchain commit latency, and RSU handover performance (count and delay).

## 2. Density and Speed Impact (SUMO Mobility)

Vehicle densities tested: [10, 30, 50, 80]. Speeds tested: [8.3, 13.9, 22.2] (m/s).

Across the tested grid, normalized PDR remained in the range 0.815–0.972. Average delay remained low (approximately 0.000770–0.000781 seconds). Throughput scaled with network load (60016–441936 bps).

Table 1: Mean±Std summary (subset shown). Full table is saved in results_pack.

| nVehicles | speedTag | pdr_norm_mean | pdr_norm_std | avgDelay_s_mean | avgDelay_s_std | throughput_bps_mean | throughput_bps_std |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10.0 | 8.3 | 0.97151 | 0.0 | 0.000781102 | 0.0 | 60016.0 | 0.0 |
| 10.0 | 13.9 | 0.97151 | 0.0 | 0.000781102 | 0.0 | 60016.0 | 0.0 |
| 10.0 | 22.2 | 0.97151 | 0.0 | 0.000781102 | 0.0 | 60016.0 | 0.0 |
| 30.0 | 8.3 | 0.849691 | 0.0 | 0.000772635 | 0.0 | 169136.0 | 0.0 |
| 30.0 | 13.9 | 0.849691 | 0.0 | 0.000772644 | 1.3416407864987245e-08 | 169136.0 | 0.0 |
| 30.0 | 22.2 | 0.849691 | 0.0 | 0.000772641 | 1.3416407864966033e-08 | 169136.0 | 0.0 |
| 50.0 | 8.3 | 0.827316 | 0.0 | 0.0007796168 | 4.0249223595190536e-09 | 278256.0 | 0.0 |
| 50.0 | 13.9 | 0.827316 | 0.0 | 0.0007796222 | 7.529940238805373e-09 | 278256.0 | 0.0 |
| 50.0 | 22.2 | 0.827316 | 0.0 | 0.0007796204 | 4.9295030175672025e-09 | 278256.0 | 0.0 |
| 80.0 | 8.3 | 0.814995 | 0.0 | 0.0007704954 | 4.669047011978836e-09 | 441936.0 | 0.0 |
| 80.0 | 13.9 | 0.814995 | 0.0 | 0.0007704966 | 6.542170893523059e-09 | 441936.0 | 0.0 |
| 80.0 | 22.2 | 0.814995 | 0.0 | 0.0007705022 | 6.496152707558068e-09 | 441936.0 | 0.0 |

## 3. Security Stress Tests (Malicious Rate + Threshold Trade-offs)

We sweep maliciousRate and trust thresholds (fast/min) to analyze robustness and access control behavior. This provides a trade-off surface between availability (FAST authentication) and security (rejecting low-trust nodes).

## 4. Figure Captions

Figure 1. SUMO density-speed: delay_vs_density_speed.png

Figure 2. SUMO density-speed: handover_count_vs_density_speed.png

Figure 3. SUMO density-speed: handover_delay_vs_density_speed.png

Figure 4. SUMO density-speed: ledger_trust_vs_density_speed.png

Figure 5. SUMO density-speed: pdr_norm_vs_density_speed.png

Figure 6. SUMO density-speed: throughput_vs_density_speed.png

Figure 7. Final sweep: delay_vs_maliciousRate_nveh_10.png

Figure 8. Final sweep: handoverCount_vs_maliciousRate_nveh_10.png

Figure 9. Final sweep: heatmap_handoverCount_thresholds.png

Figure 10. Final sweep: heatmap_ledgerTrust_thresholds.png

Figure 11. Final sweep: heatmap_pdr_norm_thresholds.png

Figure 12. Final sweep: heatmap_rejectCount_thresholds.png

Figure 13. Final sweep: ledgerTrust_vs_maliciousRate_nveh_10.png

Figure 14. Final sweep: pdr_norm_vs_maliciousRate_nveh_10.png

Figure 15. Final sweep: rejectCount_vs_maliciousRate_nveh_10.png

Figure 16. Final sweep: throughput_vs_maliciousRate_nveh_10.png